ROAD-SHIELD

An AI-Powered Mobile Sensing Framework for Multi-Modal Road Distress Detection and Urban Safety Monitoring Using Public Transport Fleets

Milestone 3 Progress Report (Draft)

CSET485 — AI and Society  ·  Smart India Hackathon 2026, SIH26124 (Bharat Electronics Limited)

Draft of 1 October 2026. Highlighted PENDING items are filled in once the licence-plate detector finishes training.

Contents

# Project Metadata and Team Information

| Field | Detail |
| --- | --- |
| Course | CSET485 — AI and Society |
| Project type | Academic Mini Project, Milestone 3 submission |
| Hackathon alignment | Smart India Hackathon 2026, Problem Statement SIH26124 (Bharat Electronics Limited) |
| Theme | Smart Automation, Intelligent Transportation Systems, Clean Mobility |
| Team | Team ID 42 |
| Code | github.com/udbhav968-creator/SIH_PROJECT, branch feature/road-scene-perception |

Table 1. Project metadata.

| S.No. | Student | Roll Number | Branch |
| --- | --- | --- | --- |
| 1 | Udbhav (Team Lead) | S24CSEU0095 | Computer Science & Engineering |
| 2 | Yuvraj Pathak | S24CSEU1182 | Computer Science & Engineering |
| 3 | Naman Bhandari | S24CSEU0611 | Computer Science & Engineering |
| 4 | Prakhar | S24CSEU1171 | Computer Science & Engineering |
| 5 | Amarjeet Kumar | S24CSEU1968 | Computer Science & Engineering |

Table 2. Student authors.

# Abstract

Milestone 2 described ROAD-SHIELD, a system that turns city buses into a road-inspection network: a dashcam and an accelerometer on each bus, an edge computer that finds and measures road defects, and a central ledger that deduplicates reports and issues tamper-evident repair orders. Milestone 3 set out to move the system from photographs of single defects to full dashcam scenes on Indian roads, to implement the privacy and edge-transmission design stated in Milestone 2, and to measure every claim against held-out data.

This milestone adds a scene-perception layer of three separately trained YOLO detectors — traffic participants (COCO), road damage (RDD2022, Indian subset) and road markings (CDSet-3434) — served on the CPU through ONNX Runtime. The zebra-crossing detector reaches test mAP50 0.869 and, asked whether a frame contains a crossing, is correct every time it says yes (0 false alarms on 263 crossing-free frames) while finding 75.4% of crossings — 3.2 times more than the geometric method it replaces. We implemented the Milestone 2 design items that existed only on paper: DPDP privacy redaction, sub-kilobyte signed edge packets, the Butterworth IMU filter (78.7% accuracy under engine vibration versus 60.1% without it), a calibrated frame-quality gate, the fairness-constrained repair Priority Index and INT8 quantization (crossing detector 1.3× faster and 2.4× smaller for a 0.006 mAP50 cost).

Measuring the Milestone 2 claims also corrected eleven of them, listed in Section 6. The software is now covered by 178 automated tests run on every commit; two security defects in the web API were found and fixed. The road-damage detector, trained on a laptop CPU, reaches test mAP50 0.245; Section 5.2 reports it class by class, including the class it does not yet detect.

# 1. Progress Against the Milestone 3 Plan

Milestone 2 closed with a four-item plan. Table 3 reports each item and the additional work that measurement showed was necessary.

| Milestone 2 plan item | Status | Outcome |
| --- | --- | --- |
| 1. Ingest RDD2022 Indian roads | Done | 7,706 labelled Indian images downloaded and prepared (5,368 train / 1,172 val / 1,166 test); road-damage detector trained: test mAP50 0.245 (CPU run; see Section 5.2) |
| 2. Real bus sensor mounting | Not started | Requires hardware access; the IMU model now trains on real field-recorded vehicle logs (Section 3) |
| 3. INT8 model quantization | Done (CPU) | Crossing detector 61 → 48 ms, 10.0 → 4.1 MB, mAP50 0.864 → 0.858; Raspberry Pi / Jetson benchmarks not yet run |
| 4. Live command-centre connection | Partial | Ranked repair ledger and scene-detection page served by the API; no live bus telemetry feed yet |
| Added: zebra-crossing detection | Done | Test mAP50 0.869; see Section 5.1 |
| Added: DPDP redaction, edge packets, IMU filter, frame gate, Priority Index | Done | Stated in Milestone 2 but not implemented; now implemented and tested |
| Added: licence-plate detector for redaction | Ready to train | Dataset and configuration prepared; model PENDING training |

Table 3. Milestone 3 plan items and outcomes.

# 2. System Architecture

ROAD-SHIELD now has two analysis paths behind one server. The inspection pipeline from Milestone 2 turns a single road photograph into a classified, measured and costed repair record. The new scene-perception layer analyses a whole dashcam frame and finds everything in it at once.

## 2.1 Scene perception

- Frame-quality gate. Rejects frames with a covered lens, darkness, glare or heavy defocus before any detector runs, saving edge CPU (Section 4.2).

- Three detectors, kept separate on purpose. Each training set labels only its own classes; a single merged model would learn that every unlabelled pothole in a crossing photograph is background. Separate models can be retrained, versioned and rolled back independently.

- Traffic: COCO-pretrained YOLO — pedestrians, cars, buses, trucks, two-wheelers, bicycles, traffic lights, stop signs and stray animals.

- Road damage: YOLO11n trained on RDD2022 India — longitudinal (D00), transverse (D10) and alligator (D20) cracks and potholes (D40).

- Road markings: YOLO11n trained on CDSet-3434 — zebra crossings and lane guide arrows.

- Scene facts and alerts. Geometric rules over the detected boxes: road users standing on a crossing (critical alert), potholes in the bus's path, vehicles blocking a crossing. Every fact states the rule that produced it.

- Privacy head. A licence-plate detector used only to blur plates; plate boxes never appear in results, APIs or transmitted packets.

## 2.2 From bus to ledger

For each analysed frame the bus sends one signed JSON packet of at most 1,024 bytes (Section 4.5). The server verifies the signature, deduplicates reports within 8 m using the Haversine distance, and orders the repair queue by the Priority Index (Section 4.6). Work orders keep the Milestone 2 SHA-256 seal.

# 3. Datasets and Data Hygiene

| Dataset | Use | Size used | Licence | Hygiene applied |
| --- | --- | --- | --- | --- |
| RDD2022, India subset (Arya et al.) | Road-damage detection | 7,706 images; 3,550 training boxes | CC BY-SA 4.0 | Clean-road images capped at one third of the training list; transverse cracks are rare (50 boxes), reported per class |
| CDSet-3434 (Zhang et al.) | Zebra crossings, guide arrows | 3,388 frames kept of 4,850 | Apache-2.0 | Official split is leaky (Section 3.1); re-split by time block with a 30-frame purge margin |
| Vehicle Registration Plates (Roboflow) | Plate redaction | 8,823 images | CC BY 4.0 | 3,904 synthetic rendered plates (44%) used for training only; val/test are real photographs |
| DNIT highway corpus (Brazil) | Distress classifier, segmenter | 1,921 crack and 564 pothole polygons | Public | Split by source photograph (unchanged from Milestone 2) |
| Pothole-Project IMU logs (India) | Shock classifier | 852 windows (688 train / 164 held-out) | Public repository | Real field recordings; time-block split with guard gap |

Table 4. Datasets used in Milestone 3.

## 3.1 A leakage finding in CDSet

All 3,434 labelled CDSet frames come from only three dashcam videos, and every frame in the official test split is one or two frames away from a training frame. Adjacent video frames are nearly identical, so any score on the official split measures memorisation. We re-split by contiguous 300-frame time blocks per video, discarded 524 frames within 30 frames of a split boundary, and kept 938 crossing-free frames out of training so they could measure false alarms. All results in Section 5.1 use this split.

## 3.2 Synthetic data in the plate dataset

44% of the licence-plate images are computer-rendered plates. They help a detector learn to localise plates, but they are easy; a test score that included them would overstate real-world performance. They are therefore confined to the training split.

# 4. Methods

## 4.1 Detector training protocol

Every shipped detector is produced by one script (training/train_detector.py) from one configuration file, so a weight file traces back to its data and settings. Training starts from COCO-pretrained YOLO11n. On a laptop CPU the learning-rate schedule is fitted to a wall-clock budget; on a GPU a fixed epoch count is used. After training, the script: (1) chooses a per-class confidence threshold at the F1 peak on the validation split; (2) scores box mAP on the test split, which the thresholds never saw; (3) scores image-level questions ("does this frame contain a crossing?") and the false-alarm rate on clean frames; (4) exports ONNX and writes a model card with the SHA-256 of each file. An interrupted run resumes from its last completed epoch.

## 4.2 Frame-quality gate

Milestone 2 proposed skipping frames whose Laplacian variance is below 42.5. Measured on 450 real road frames from three camera sources, that threshold would have discarded 10.4% of good frames, because texture varies ten-fold between cameras (median 1,192 on CDSet dashcam frames, 127 on the project photographs). The gate instead combines three calibrated checks on the road region: Laplacian variance below 5.0, mean luminance below 15, or more than half the pixels saturated.

## 4.3 IMU band-pass filter

Accelerometer windows pass through a 4th-order Butterworth band-pass filter (0.5–25 Hz, zero phase) as the first step of the saved model, so training and deployment cannot disagree. We adopted it only after measuring it (Section 5.3).

## 4.4 Privacy redaction (DPDP Act 2023)

Every detected person is blurred before an image leaves the system; images produced by the server and command-line tools are redacted by default. Plates are blurred by the dedicated plate detector at a recall-leaning threshold of 0.15, because a missed plate costs far more than blurring a false positive. Until that model is trained the output states explicitly that plates were not redacted.

## 4.5 Edge event packet

A bus transmits, per analysed frame, a JSON packet of at most 1,024 bytes: road findings with class code, confidence and normalised box; counts (never boxes) of people and vehicles; and alerts. When a crowded frame would exceed the limit, the least important findings are dropped first (potholes are kept longest) and the number dropped is recorded. Packets carry an HMAC-SHA256 keyed per bus: unlike a bare SHA-256, it cannot be recomputed by someone who alters the packet.

## 4.6 Repair Priority Index

The Milestone 2 index PI = w1(100 − PCI) + w2·Vol + w3·Traffic is implemented with each term scaled to [0, 1] so that the weights mean what they say (defaults 0.5, 0.2, 0.3):

PI = 100 · [ 0.5·(100 − PCI)/100 + 0.2·V/(V + 0.06) + 0.3·log(1 + AADT)/log(1 + 100,000) ]

Volume saturates (a 2 m³ crater is not forty times more urgent than a small pothole) and traffic is logarithmic. The function accepts only measured road quantities; it has no input for ward, locality or number of complaints, and an automated test confirms that two defects with identical measurements in a VIP area and an outer colony receive identical priority. When traffic has not been measured, the traffic term is dropped and the weights renormalised rather than guessed.

## 4.7 INT8 quantization

Models are quantized with static, per-channel INT8 quantization calibrated on training images only and scored against the full-precision model on the same held-out images. For YOLO detectors the final detection block is kept in full precision: it concatenates box coordinates (0–416 pixels) with class probabilities (0–1), and a single INT8 scale for both collapsed the crossing detector's mAP50 from 0.864 to 0.100 in our first attempt.

# 5. Results

All figures below are measured on held-out data that the models and thresholds never saw. Hardware: Intel Core i5-1145G7 laptop CPU, no GPU.

## 5.1 Zebra-crossing detector

| Class | Precision | Recall | mAP50 | mAP50-95 |
| --- | --- | --- | --- | --- |
| Zebra crossing | 0.968 | 0.761 | 0.878 | 0.549 |
| Lane guide arrow | 0.807 | 0.785 | 0.861 | 0.599 |
| All classes | 0.888 | 0.773 | 0.869 | 0.574 |

Table 5. Box-level test metrics (620 frames; YOLO11n, 416 px, 12 epochs, ~2.1 h CPU training).

| Does the frame contain a zebra crossing? | Learned detector | Geometric method (Milestone 2) |
| --- | --- | --- |
| Precision | 1.000 | 0.933 |
| Recall | 0.754 | 0.235 |
| F1 | 0.859 | 0.376 |
| False alarms on 263 crossing-free frames | 0 (0.0%) | 6 (2.3%) |
| Median latency per frame | 81 ms | 17 ms |

Table 6. Head-to-head on the same 620 held-out frames, scored through the deployed ONNX model.

## 5.2 Road-damage detector

| Class | Precision | Recall | mAP50 | mAP50-95 |
| --- | --- | --- | --- | --- |
| Longitudinal crack (D00) | 0.354 | 0.201 | 0.192 | 0.076 |
| Transverse crack (D10) | — (no detections) | 0.000 | 0.000 | 0.000 |
| Alligator crack (D20) | 0.444 | 0.598 | 0.543 | 0.244 |
| Pothole (D40) | 0.328 | 0.313 | 0.244 | 0.125 |
| All classes | 0.531 | 0.278 | 0.245 | 0.111 |

Table 7. Road-damage test metrics on 1,166 held-out Indian images (YOLO11n, 512 px, 16 epochs, 6.1 h on a laptop CPU). Serving thresholds tuned on the validation split: longitudinal crack 0.156, transverse crack 0.05, alligator crack 0.326, pothole 0.17.

These figures come from a short CPU training run (16 epochs at 512 px) and are a baseline. Validation mAP50 was still rising when the time budget ended (0.253 at epoch 15, 0.257 at epoch 16), so the model is under-trained; we have not measured how much longer training on a GPU improves it. Results differ sharply by class. Alligator cracks are detected best (mAP50 0.543); potholes and longitudinal cracks are fair; transverse cracks are not detected at all — the Indian subset gives only 50 training and 7 test boxes for that class, so its row is not a stable measurement. At image level the detector answers "does this frame show a pothole?" with precision 0.431 and recall 0.503, and raises an alarm on 16.0% of clean-road frames. It is a screening aid for routing a survey crew, not yet a substitute for one.

## 5.3 IMU band-pass filter

| Condition (164 held-out windows, 5 seeds) | Without filter | With 0.5–25 Hz filter |
| --- | --- | --- |
| Clean recordings | 83.7% | 81.5% |
| 35 Hz engine vibration added | 60.1% | 77.2% |
| Engine vibration + 12° tilted mount | 60.9% | 78.0% |

Table 8. The filter costs about two points on clean logs and gains about seventeen under the vibration a bus-mounted sensor experiences. The retrained production model scores 80.5% clean and 78.7% with vibration.

## 5.4 Frame-quality gate

| Frames | Kept / caught |
| --- | --- |
| 450 real road frames (three camera sources) | 100% kept |
| Covered lens | 100% caught |
| Severe defocus | 100% caught |
| Whiteout / sun glare | 98% caught |
| Moderate defocus | 89% caught |
| Horizontal motion blur | 4% caught (known limitation) |

Table 9. Gate calibration (scripts/calibrate_frame_gate.py).

## 5.5 INT8 quantization

| Model | Metric FP32 → INT8 | Latency | Size |
| --- | --- | --- | --- |
| Zebra-crossing detector | mAP50 0.864 → 0.858 | 61 → 48 ms (1.3×) | 10.0 → 4.1 MB |
| Road-damage detector | mAP50 0.234 → 0.222 | 86 → 75 ms (1.1×) | 10.1 → 4.2 MB |
| Classifier backbone (MobileNetV2) | Accuracy 93.4% → 90.8% | 21 → 12 ms (1.8×) | 13.3 → 3.7 MB |

Table 10. INT8 results (percentile calibration). The detector loses almost nothing; the classifier loses 2.6 points, so full precision remains the default and INT8 is offered for low-power edge boards. Detector mAP50 values here are scored through the ONNX files at batch size 1, so they differ slightly from Tables 5 and 7; the FP32-to-INT8 difference is the measurement. Calibration used 128 images (48 for the road-damage detector, which ran out of memory with more). The classifier figures compare FP32 and INT8 on identical images; because the DNIT crops were regenerated, that image set is not exactly the published held-out split, so the absolute 93.4% is not comparable to the published 90.1% — the 2.6-point difference is the measurement.

## 5.6 Latency

On a 1280x720 frame (laptop CPU, median of 15 runs, including the quality gate): traffic 203 ms, damage 115 ms, markings 76 ms, all three 391 ms; classifier backbone 21 ms.

# 6. Corrections to the Milestone 2 Report

Checking each Milestone 2 claim against the code and measurements produced the following corrections. We report them in the same spirit as Milestone 2's disclosure on macro-F1: a number that cannot be reproduced should not be cited.

| # | Milestone 2 stated | Measured / actual |
| --- | --- | --- |
| C1 | Classifier detects 9 classes | 7 classes; the new detectors add 4 damage, 2 marking and COCO traffic classes |
| C2 | 10/10 subsystem tests pass | 178 automated tests, run by CI on Python 3.11 and 3.12 |
| C3 | IMU telemetry simulated (spring-mass-damper); 15,000 windows | 852 real field-recorded windows; 80.5% held-out |
| C4 | Gatekeeper threshold: Laplacian variance 42.5 | Would drop 10.4% of real frames; calibrated gate keeps 100% |
| C5 | UrbanTrafficNet 90.36% accuracy | Not reproducible — no such trained network exists; counts now come from the COCO detector |
| C6 | Faces and plates blurred on the bus | People blurred; plates once the plate detector is trained |
| C7 | Full inference cycle 76 ms | Traffic detector 202 ms; crossing 76 ms; backbone 21 ms |
| C8 | Camera height 2.45 m | Calibration profiles use 1.45–1.52 m |
| C9 | ResNet-50 is the served backbone | A fresh installation serves MobileNetV2 (90.1%); ResNet-50 (89.2%) is an optional download |
| C10 | RDD2022 India: 1,000+ frames; 47,000 photos to ingest | 7,706 labelled Indian images used; 47,420 is all six countries |
| C11 | Deduplication within 8 m | The running server used 10 m; corrected to 8 m |

Table 11. Corrections. Evidence for each is in docs/REPORT_ALIGNMENT.md.

# 7. Software Engineering and Security

- Testing. 178 automated tests, including the web API exercised over real HTTP, per-model integration tests that verify the deployed model file against the SHA-256 in its model card, and fairness and privacy properties.

- Security. The API opened any file on the server whose path was sent in an image field, and accepted a path to any file on the machine. Image fields are now decoded strictly as data, paths are confined to the dataset folder, and request sizes are capped.

- Dependencies. Model files had been saved with a newer scikit-learn than the requirements allowed, so CI loaded them on a different version than the one they were trained with; the pin was corrected.

- Reproducibility. Every figure in this report is regenerated by a script named in docs/REPORT_ALIGNMENT.md; detector training also runs in a provided Google Colab notebook.

- Licensing. Ultralytics YOLO is AGPL-3.0 and RDD2022 is CC BY-SA 4.0 (share-alike); both affect any deployment beyond academic use and are documented in the README.

# 8. Limitations and Ethical Considerations

- Accelerometer logs come from a road vehicle, not yet a bus; the vibration test simulates engine vibration rather than recording it.

- Zebra-crossing data comes from three videos; performance on Indian crossings with faded paint is not yet measured.

- The road-damage detector is under-trained (validation mAP50 still rising at the last epoch) and does not detect transverse cracks, for which the Indian subset has 50 training boxes.

- Redaction blurs whole person boxes rather than faces, and plate redaction depends on a detector still to be trained.

- Horizontal motion blur mostly passes the frame-quality gate.

- No demographic inference is made about people in frame, by design; people are only counted and blurred.

- The Priority Index removes locality from the decision, but its weights are a policy choice and should be set with municipal stakeholders.

# 9. Next Steps

- Train the licence-plate detector and retrain the road-damage detector for 100 epochs at 640 px (GPU notebook provided).

- Record accelerometer data on an operational bus and retrain the shock classifier.

- Benchmark the INT8 models on a Raspberry Pi 5 and an NVIDIA Jetson.

- Collect Indian zebra-crossing footage to measure the crossing detector on local road markings.

- Connect live bus telemetry to the command-centre map.

# References

[1] D. Arya, H. Maeda, S. K. Ghosh, D. Toshniwal and Y. Sekimoto, "RDD2022: A multi-national image dataset for automatic road damage detection," Geoscience Data Journal, vol. 11, no. 4, pp. 846–862, 2024.

[2] Z.-D. Zhang, M.-L. Tan, Z.-C. Lan, H.-C. Liu, L. Pei and W.-X. Yu, "CDNet: A real-time and robust crosswalk detection network on Jetson nano based on YOLOv5," Neural Computing and Applications, vol. 34, no. 13, pp. 10719–10730, 2022.

[3] G. Jocher et al., Ultralytics YOLO (YOLO11), software, AGPL-3.0, 2024–2026.

[4] Microsoft, ONNX Runtime quantization documentation, 2026.

[5] Augmented Startups, "Vehicle Registration Plates Dataset," Roboflow Universe, CC BY 4.0, 2022.

[6] J. Eriksson et al., "The Pothole Patrol: using a mobile sensor network for road surface monitoring," in Proc. ACM MobiSys, 2008.

[7] A. Mednis et al., "Real time pothole detection using Android smartphones with accelerometers," in Proc. MobiSensor, 2011.

[8] ASTM International, ASTM D6433-24: Standard Practice for Roads and Parking Lots Pavement Condition Index Surveys, 2024.

[9] Ministry of Road Transport and Highways, Specifications for Road and Bridge Works (5th Revision), Section 500, Indian Roads Congress.

[10] Digital Personal Data Protection Act, 2023, Government of India.

[11] Smart India Hackathon 2026, Problem Statement SIH26124, Bharat Electronics Limited.