REPORT: -
ROAD-SHIELD: AN AI-POWERED MOBILE SENSING FRAMEWORK FOR MULTI-MODAL ROAD DISTRESS DETECTION AND URBAN SAFETY MONITORING USING PUBLIC TRANSPORT FLEETS

Project Metadata and Team Information

Course Code & Name: CSET485 — AI and Society

Project Type: Academic Mini Project (Phase 2 / Milestone 2 Submission)

Hackathon Alignment: Smart India Hackathon 2026 (Problem Statement: SIH26124 / Bharat Electronics Limited)

Theme: Smart Automation, Intelligent Transportation Systems (ITS) & Clean Mobility

Submission Date: September 2026

Team Identifier: Team ID: 42

Student Authors and Credentials

| S.No. | Student Full Name | University Enrolment / Roll Number | Department / Branch |
| --- | --- | --- | --- |
| 1 | Udbhav (Team Lead) | S24CSEU0095 | Computer Science & Engineering |
| 2 | Yuvraj Pathak | S24CSEU1182 | Computer Science & Engineering |
| 3 | Naman Bhandari | S24CSEU0611 | Computer Science & Engineering |
| 4 | Prakhar | S24CSEU1171 | Computer Science & Engineering |
| 5 | Amarjeet Kumar | S24CSEU1968 | Computer Science & Engineering |

ABSTRACT: -

Anyone who travels on Indian roads knows firsthand how dangerous potholes and broken surfaces are. Official data from the Ministry of Road Transport and Highways (MoRTH) showed that between 2018 and 2020, over 5,600 people died in crashes directly linked to potholes, with two-wheeler riders taking the heaviest hit. Right now, local municipalities and PWD offices still depend almost entirely on manual walking surveys or occasional inspections to find road damages. This process is very slow, happens in scattered patches, and depends heavily on who is doing the survey. On top of that, paper records can easily be altered, leading to inflated repair bills and poor accountability. High-tech laser survey vans exist, but at $150,000 to $500,000 per van, no municipal budget can afford to run them across every city street on a daily basis.

To solve this practically, our team developed ROAD-SHIELD for Smart India Hackathon problem statement SIH26124 (Bharat Electronics Limited). Our core idea is straightforward: why buy dedicated survey vans when city buses are already driving along these roads every single day? The system is designed to run on an ordinary dashcam and an inexpensive MPU-6050 accelerometer (100 Hz) mounted on city transit buses; no bus has been instrumented yet, so images come from public road datasets and accelerometer data from real car drive logs. Instead of streaming heavy raw video over cellular networks—which drains bandwidth and invades pedestrian privacy—the software is written to run on an on-board edge computer (it currently runs on a laptop-class CPU). It uses an ImageNet-pretrained MobileNetV3-Large fine-tuned end to end on our road photographs, run on CPU via ONNX Runtime to sort each region into 7 road-condition classes, estimates their real-world area by projecting pixels onto the road plane (from an assumed camera mount unless a per-vehicle calibration profile is supplied) and their depth as an interval estimate rather than a measurement, checks, when an accelerometer window is supplied, whether the vehicle actually felt a bump using a Bayesian sensor gate, and scores the pavement on ASTM D6433 standards. It then estimates repair bitumen tonnage using MoRTH Section 500 schedules and locks each work order with a SHA-256 hash to stop contractor tampering. On our held-out validation photos, our classifier scored 93.3% accuracy (macro-F1 0.828) on 630 images from 567 photographs it never saw, the hand-crafted baseline scored 84.9% on the same split, and the code has all 209 automated tests passing (1 skipped by design). On 1200 crops from held-out RDD2022 India photographs (split by photograph; no training run reads them), the classifier scored 92.3% accuracy (macro-F1 0.922).

INTRODUCTION: -

Road damage in India is not just an inconvenience—it is a life-threatening safety crisis. Between 2018 and 2020, government records listed 2,015 pothole deaths in 2018, 2,140 in 2019, and 1,471 in 2020. With urban traffic climbing each year, the real count on city streets is likely higher. The danger peaks during the monsoon season. Rainwater fills up holes, hiding their true depth from drivers and bikers, while water seeping under the asphalt rapidly loosens the road foundation. Tragic cases, like recent fatal accidents in Noida and Delhi where commuters fell into flooded craters, happen because authorities simply do not find or fix defects before tragedy strikes.

While ASTM D6433 sets standard procedures for calculating Pavement Condition Index (PCI) ratings, doing this manually requires engineers with measuring tapes walking lane by lane, which takes weeks and cannot cover an entire city. Smartphone-based apps have tried to crowdsource accelerometer data, but phone sensors move around in pockets, cars have different suspension softness, and an accelerometer alone cannot tell whether you hit a pothole, a manhole lid, or an authorized speed breaker. ROAD-SHIELD fixes this by mounting fixed, calibrated cameras and inertial sensors on city buses. Because transit buses run fixed routes continuously from early morning to late night, they act as an automatic roving inspection grid that monitors the whole city without needing extra drivers, expensive laser trucks, or separate survey runs.

DOMAIN TAXONOMY AND THE CLASSIFICATION: -

Our project brings together four key technical and societal areas:

Primary Domain: Intelligent Transportation Systems (ITS) and Civil Infrastructure Asset Management.

Secondary Domain: Computer Vision, specifically monocular depth estimation, defect classification, and Inverse Perspective Mapping (IPM).

Tertiary Domain: Edge AI and Cyber-Physical Systems, using 100 Hz tri-axial accelerometer shock fusion without heavy GPUs.

Societal Domain: Public governance, civic infrastructure auditing, and preventing contractor fraud under MoRTH/CPWD civil guidelines.

1. Problem Domain Identification and Definition: -

Our work addresses SIH26124 issued by Bharat Electronics Limited under MeitY: 'AI-assisted road quality assessment and urban infrastructure monitoring using the public bus fleet as a distributed mobile sensing network.' In plain terms, we turn raw camera images and chassis vibrations from buses into verified, un-tampered, cost-estimated road repair work orders ready for municipal engineers.

The Five Key Building Blocks:

1. Opportunistic Mobile Sensing at the Edge: We do not hire dedicated drivers. City buses already run regular routes. Adding a low-power edge computer to each bus would make inspection passive during routine transit (proposed deployment; not yet trialled on a bus).

2. Real Metric Sizing: Drawing a 2D bounding box on a screen does not tell an engineer how much asphalt is needed. We project segmented pixels onto the road plane using the camera's height, pitch and focal length to get square metres; depth cannot be measured from one image, so it is reported as an estimate with a low-high interval.

3. Multi-Bus Spatial Deduplication: If ten buses drive over the same pothole on MG Road in one morning, the city should not receive ten different repair orders. Our system merges reports of the same defect class within 8 metres (Haversine distance), confirming the defect while merging the reports into one ticket.

4. Civil Schedule of Rates (SOR) Interoperability: We take the measured defect size and calculate the bitumen tonnage and tack coat required under MoRTH Section 500 rules, giving municipal staff an instant cost estimate.

5. Civic Accountability and Privacy: To follow the DPDP Act 2023, a redaction step blurs the head region of every detected person and any number plate found inside detected vehicles before an image is shared (models/privacy_redactor.py); its recall has not yet been measured on an annotated set. To stop billing fraud, every work order is signed with a SHA-256 digital seal.

2. Comprehensive Problem Analysis: -

2.1 How Roads Actually Break: Potholes do not appear out of nowhere. Water enters hairline cracks, traffic tires press down with high hydraulic pressure, fines wash out from underneath, and the surface caves in under heavy bus axles within days.

2.2 Shortcomings of Manual Walking Surveys: Inspections are infrequent and subjective. An engineer might rate a crack as minor while another rates it as severe, and vast residential sectors get checked once a year at best.

2.3 Dedicated Survey Vans (ARAN / LiDAR): Commercial laser vans cost between ₹1.2 crore and ₹4 crore each ($150k-$500k). Few Indian municipalities have the budget to buy or operate them regularly.

2.4 Citizen Complaint Apps Fail in Practice: Apps like Meri Sadak rely on citizens reporting defects after an accident or vehicle breakdown has already occurred. Response times are slow and photos lack GPS precision.

2.5 The 2D Bounding Box Trap: Drawing a rectangle around an angled crack includes large areas of healthy asphalt, overstating repair costs up to 13 times. Also, camera-only models can easily mistake harmless oil stains or shadows for deep holes.

2.6 Ticket Flooding Across Fleets: Without spatial clustering, a busy bus corridor generates thousands of redundant alerts for the same few potholes, burying urgent municipal priorities.

2.7 Contract Manipulation and Billing Scams: Because manual repair estimates rely on handwritten paperwork, contractors can easily inflate repair dimensions or resubmit old photos to get invoices paid twice.

3. Justification Of the Problem Analysis: -

3.1 Protecting Human Lives: MoRTH recorded 5,626 pothole deaths across 2018-2020, roughly 1,900 a year. Motorcyclists, scooter riders, and cyclists have zero chassis protection, making sudden swerves or pothole impacts fatal.

3.2 Economic and Fuel Waste: Constant braking and accelerating over damaged roads burns 12% to 16% more fuel and accelerates vehicle suspension damage. Blocked lanes also delay emergency ambulances and fire trucks.

3.3 Legal Mandates and Article 21: The Supreme Court of India has repeatedly ruled that citizen safety on public roads falls under the Fundamental Right to Life (Article 21). Furthermore, Section 198A of the Motor Vehicles (Amendment) Act 2019 places direct legal liability on designated road authorities and contractors for faulty maintenance.

3.4 Multi-Modal Sensor Fusion Justification: Cameras give visual context but cannot feel depth. Accelerometers feel shocks but cannot distinguish potholes from speed breakers. Combining both eliminates false alarms without needing a $200,000 laser sensor.

3.5 AI and Society (CSET485) Societal Priority Index: In many cities, VIP roads get repaved instantly while crowded outer colonies are neglected. We propose an objective Priority Index: PI = w1*(100 - PCI) + w2*Vol + w3*Traffic. It is not implemented yet and its weights are not set; the current ledger orders work by PCI-based SLA tiers (24 / 48 / 72 hours).

4. Literature Review: -

We conducted a detailed review of 12 key research efforts to identify where prior systems fell short and how ROAD-SHIELD improves upon them:

| # / Author & Year | Methodology & Tech Stack | Key Contribution | Identified Weakness / Failure Mode | How ROAD-SHIELD Solves This |
| --- | --- | --- | --- | --- |
| 1. Eriksson et al. (2008) ACM MobiSys | Boston taxi fleet with external 3-axis accelerometers and GPS; heuristic Z-peak filtering. | First large vehicular crowdsensing system (The Pothole Patrol); found 90% of severe holes. | Huge false-alarm rate: cannot distinguish potholes from manhole lids, joints, or railroad tracks. | Bayesian Dual-Sensor Gating: requires visual confirmation from dashcam before flagging a chassis shock. |
| 2. Koch & Brilakis (2011) Adv. Eng. Informatics | Monocular RGB edge detection, thresholding, and morphological region growing on asphalt. | Detected potholes in asphalt without needing neural network training; reached 86% accuracy (82% precision, 86% recall) on 120 images. | Extremely sensitive to light: fails when tree shadows fall on the road, on wet pavement, or near fresh blacktop. | Asphalt Gatekeeper + CNN: normalizes contrast and uses an ImageNet MobileNetV3-Large fine-tuned end to end; 93.3% held-out accuracy. |
| 3. Mednis et al. (2011) IEEE DCOSS | Android phones inside cars running 4 heuristic accelerometer threshold algorithms (Z-THRESH, G-ZERO). | Demonstrated that consumer smartphones can detect road anomalies; reported true-positive rates up to 90% (Z-DIFF). | Phone position drifts inside car; vehicle suspension differences skew threshold readings; zero lane tracking. | Fixed-mount fleet hardware (proposed): camera and IMU bolted to the bus, with a per-vehicle calibration profile. |
| 4. Arya et al. (2020/2022) IEEE Big Data | Global Road Damage Dataset (RDD2020/2022, 47k images); YOLOv4/v5 multi-class bounding boxes. | Established international benchmark classes for road distress (D00, D10, D20, D40). | 2D Bounding Box Trap: drawing rectangular boxes over diagonal cracks overestimates repair area up to 13-fold. | Pixel-level Defect Homography: maps true defect contours to real metric ground area (m^2) via calibrated IPM. |
| 5. Fan et al. (2019/2020) IEEE T-IP | Stereo-vision depth disparity maps; transformed disparity analysis for cavity extraction. | Created dense 3D road depth profiles without laser tools, achieving millimeter-level depth accuracy. | High compute & vibration fragility: dual cameras easily lose calibration on bumpy buses; requires high-power GPUs. | Monocular ground-plane projection + IRC depth bands: area from one calibrated camera, depth as an interval within IRC patching bands; runs on CPU (power draw not measured). |
| 6. Zhu et al. (2022) Autom. in Constr. | UAV imagery with YOLO-family detectors for highway pavement distress. | Aerial inspection of highway pavements from drone imagery. | Impractical for cities: drones cannot legally fly over crowded city streets, flight time is only 30 mins, fails in wind. | Public Transit Bus Integration: zero flight restrictions; operates all day long across 85%+ of arterial roads. |
| 7. Bhatia et al. (2022) J. King Saud Univ. CIS | Thermal images classified with CNNs to detect potholes. | Detects potholes in thermal images, which do not depend on daylight. | Prohibitive sensor cost: thermal cameras cost 10x-20x more than regular dashcams and get blinded in hot tropical sun. | Monsoon Degradation Forecaster: uses a 180-day mathematical curve based on rainfall and axle loads instead of thermal cameras. |
| 8. Yang et al. (2024) NeurIPS | Depth Anything V2 monocular metric depth estimation using massive synthetic pre-training. | Zero-shot depth estimation from a single camera image with clean, crisp edges. | Severe latency and scale drift: needs 8 GB+ VRAM GPU; estimated metric depth shifts whenever vehicle pitches. | Per-Vehicle Calibration Tables: explicit focal length and camera pitch lookup tables keep metric sizing consistent. |
| 9. Kirillov et al. (2023) Meta AI | Segment Anything Model (SAM) promptable foundation model for zero-shot object segmentation. | Industry benchmark for detailed mask boundaries on any general visual object. | Massive model size: 636M parameters; takes ~2.5 seconds per frame, completely unusable on low-power vehicle hardware. | Lightweight 11-Feature Classifier: evaluates candidate patches in milliseconds on CPU with solid boundary overlap. |
| 10. ASTM D6433 Standard (2020/2024) | Standardized manual deduct-value curves calculating Pavement Condition Index (PCI: 0 to 100). | Recognized official civil engineering standard for road quality assessment worldwide. | Completely manual: requires technicians to measure defects with rulers and look up paper charts; cannot run live. | Automated M_PCI Deduct Engine: codified ASTM deduct curves directly into Python algorithms for instant scoring. |
| 11. MoRTH Section 500 Specification | Official Indian government specifications for bituminous base and surface repairs, materials, and rates. | The statutory legal basis for all government road contracting and financial spending in India. | Completely paper-based: exists only as printed books; contractors calculate bitumen tonnage by hand. | Civil Volumetric Ledger: directly converts detected defect volume V = double_int(z dx dy) to MoRTH tonnage and costs in INR. |
| 12. Anjomshoaa et al. (2018) IEEE IoT J. | City Scanner: drive-by sensing platform mounted on municipal garbage trucks. | Proved that utility vehicles already on the road provide cheap, widespread urban coverage. | Environmental sensing platform: no defect deduplication or tamper-evident work orders. | Haversine Clustering + SHA-256: merges duplicate sightings within 8 m and seals work orders with SHA-256 hashes. |

5. Datasets & Data Hygiene Architecture: -

To train and test ROAD-SHIELD thoroughly, we combined several public datasets, real-world road photos, and inertial telemetry streams:

| Dataset & Source | Modality & Res | Samples Kept | Target Classes | Known Gaps & Biases | Data Hygiene Applied |
| --- | --- | --- | --- | --- | --- |
| DNIT Highway Corpus (Brazil) | RGB (1920x1080) | 1,667 images (1,200 crack, 465 pothole) | Long./Trans./Alligator Cracks, Potholes | Smooth highway asphalt; lacks Indian street clutter (e.g. rickshaws, debris). | Split strictly by original photograph so augmented crops never cross between train and test. |
| RDD2022 India (Arya et al., CRDDC 2022) | Smartphone Dashcam (720p/1080p) | Training: 1200 crack, 1200 pothole, 600 normal-road crops from 1785 photographs; Indian test set: 1200 crops from 781 photographs | Sound Road, Cracks (D00/D10/D20), Potholes (D40) | Severe class skew; annotated with bounding boxes rather than segmentation masks; smartphone/car imagery, not bus-mounted. | Split by photograph: train+valid photographs give training crops (prefixed rddin_), test photographs go only to datasets/_eval_rdd2022_india, which no training run reads. (The folder named 01_rdd2022_india holds DNIT copies, not RDD data, and is not used.) |
| Kaggle Pothole & Plain Road Collection | RGB (Various resolutions) | 1,287 files (915 pothole, 372 plain road); 700 of 739 images in one re-uploaded set rejected as pHash duplicates | Normal Road, Potholes | High duplicate redundancy; 94% of images were identical re-uploads. | 64-bit DCT perceptual hash (pHash) flagged and removed 700 duplicates (Hamming dist <= 8 bits). |
| Kaggle Surface Concrete Cracks | Close-range RGB (227x227) | 2,376 patches - excluded from training | Micro crack texture | Close-range concrete surfaces, not open asphalt road scenes. | Excluded from road-scene training and measurement by pipeline/corpus_policy.py (227x227 wall/concrete close-ups with no road geometry). |
| CRACK500 (Yang et al.) | Pavement RGB (2000x1500) | None - not in the corpus | Severe fatigue and alligator cracking | Uniform laboratory lighting; lacks shadows or wet road reflections. | No CRACK500 image is in the repository; the folder named 03_crack500_fatigue holds DNIT road photos and the excluded Kaggle concrete patches. Listed here only to correct earlier drafts. |
| Urban Hazard Set (Wikimedia / Geograph) | Field Telephoto RGB | 207 images: 67 field photographs (14/19/20/14) + 140 Wikimedia Commons (35 per class) | Waterlogging, Missing Zebra, Missing Divider, Damaged Signs | Extreme real-world scarcity; small sample size yields sensitive F1 scores. | Manually audited and cleaned; 299 intact road signs incorrectly filed as 'damaged' were removed. |
| Tri-Axial IMU Telemetry (Sensor Suite) | 100 Hz 3-Axis Accelerometer | 852 windows (1.0 s): 688 train, 164 held out, from 10 real drive logs (205,501 rows) | Smooth Asphalt, Unmarked Speed Breaker / Bump, Marked Speed Breaker, Pothole Impact | Real field recordings from Indian road drives (car, not bus) published by one GitHub project; a small set of 10 logs, split by time block (first 80% / last 20% of each log). | Extracted FFT spectral power, crest factor, and peak acceleration to train the M4 shock model. |

6. Project Feasibility Assessment: -

6.1 Technical Feasibility: Our models run on standard vehicle CPUs using ONNX Runtime, classifying an image region in about 7 ms without needing a GPU; the full per-photograph pipeline (segmentation, semantic gate, classification, costing) averaged 2.6 s on a 2-thread CPU.

6.2 Operational Feasibility: We attach sensors to buses that run anyway. Drivers do not have to touch the system, and routes do not change. Everything operates autonomously in the background.

6.3 Economic Feasibility: A single commercial laser van costs upwards of ₹1.5 crore ($150,000+). ROAD-SHIELD uses a standard dashcam and an MPU-6050 board costing less than ₹15,000 per bus, cutting equipment costs by over 95%.

6.4 Legal & Privacy Feasibility: People's heads and number plates are blurred by the redaction step before an image is shared, in support of the DPDP Act 2023; detection recall has not yet been measured. Meanwhile, SHA-256 digital seals on work orders provide tamper-proof evidence for legal accountability.

7. Preprocessing Techniques & Feature Engineering: -

Because our system runs on an edge computer inside a moving bus, our preprocessing steps are designed to be fast and lightweight:

7.1 Standardizing Frame Dimensions: All incoming video frames are converted to 640x480 pixels and normalized with ImageNet mean and variance channel values.

7.2 Asphalt-Texture Gatekeeper: There is no point running deep models when the camera sees sky, rain wipers, or the back of a truck. We calculate the standard deviation of luminance over the lower 65% of the frame. If it is below 6.5 (no visible asphalt grain), the frame is flagged as non-road and skipped immediately, saving CPU power.

7.3 Inverse Perspective Mapping (IPM) Homography: Road cameras look forward at an angle, making nearby potholes look huge and distant ones look tiny. Using the camera height and pitch (1.45 m / 18.4 deg assumed by default; 1.52 m / 16 deg in the one bus profile, whose intrinsics come from the lens field of view) and the focal length, every defect pixel is projected onto the flat road plane and its ground footprint summed, giving square metres of damage.

7.4 100 Hz Accelerometer Bandpass Filtering: Raw vertical acceleration has constant 1.0g gravity and engine vibration mixed in. Each 1-second window has its per-axis mean removed (which removes gravity and mounting tilt); the RandomForest then uses FFT band energies, crest factor, skewness, kurtosis and jerk, and the 1-D CNN learns directly from the centred signal. No band-pass filter is applied.

7.5 DPDP Act 2023 Privacy Blurring: A redaction pass blurs the head region of every detected person and any plate the contour localiser finds inside detected vehicles (Gaussian kernel of at least 15x15 px, scaled to the region). Work orders are compact JSON sealed with SHA-256, which makes them tamper-evident but not encrypted; transport encryption (TLS) belongs to the deployment and is not implemented in this prototype.

8. Proposed Methodology & Complete Framework: -

ROAD-SHIELD processes sensor data through an eleven-stage pipeline connecting on-vehicle sensing with the municipal GIS center:

Stage 1: Optical Frame Decoding: Standardizes incoming dashcam frames to 640x480 resolution.

Stage 2: Asphalt-Texture Gatekeeper: Calculates luminance standard deviation over the road region and filters out frames that do not depict a road surface.

Stage 3: Defect Proposal Region Isolation: Finds candidate distress regions with an 11-feature pixel segmenter (gradient-boosted trees), whose pothole pixels are kept only where the CNN classifier also sees a pothole (semantic gate); painted markings are masked out first.

Stage 4: VisionDistressNet Classification: An ImageNet-pretrained MobileNetV3-Large, fine-tuned end to end on the road corpus (chosen among 4 architectures on validation data, then refit on train+val), predicts across 7 distress classes, averaging each crop with its mirror. The frozen MobileNetV2 + scikit-learn head is kept as the fallback.

Stage 5: IPM Homography Defect Sizing: Converts candidate pixel regions into ground area (m^2) by ground-plane projection, and attaches a depth estimate with a low-high interval (not a measurement).

Stage 6: 100 Hz IMU Window Sync: Scores a 1.0-second tri-axial accelerometer window when one is supplied with the frame; without one, the stage reports that no IMU data was available rather than inventing it.

Stage 7: Bayesian Dual-Sensor Fusion Gate: Calculates posterior defect probability: P(Defect|Vision, Shock) = [P(Shock|Defect)*P(Defect|Vision)] / P(Evidence). If an image shows a dark patch but the accelerometer felt zero shock, confidence drops, preventing false alarms from harmless shadows or oil spills.

Stage 8: ASTM D6433 PCI Regression: Matches defect type, severity, and density against ASTM D6433 deduct curves to compute a continuous 0 to 100 condition score.

Stage 9: 180-Day Monsoon Deterioration Forecaster: Projects future structural decay over 30, 60, 90, and 180 days based on seasonal rainfall and daily bus axle loads (ESALs).

Stage 10: MoRTH Section 500 Civil Costing: Calculates required repair materials: Bitumen Mass (tons) = Area (m^2) * Depth (m) * mix density (DBM 2.40, BC 2.35, cold mix 2.20 t/m^3) * 1.15 compaction margin, computing exact bitumen and labor repair budgets in INR.

Stage 11: SHA-256 Cryptographic Sealing: Hashes all work order fields into an immutable SHA-256 digital signature to prevent contractor invoice manipulation.

At the central server level, our Haversine spatial deduplication engine checks incoming GPS reports. If different buses report the same defect within an 8-meter radius, the system clusters them into one verified hotspot instead of dispatching duplicate repair crews.

9. Relevance & Technical Justification of Algorithms/Techniques: -

Here is why we chose our specific algorithms over other common alternatives:

| Subsystem Component | Selected Technique | Evaluated Alternatives | Technical Justification & Relevance |
| --- | --- | --- | --- |
| Feature Extractor | ImageNet MobileNetV3-Large fine-tuned end to end (ONNX Runtime) | HOG+LBP; Deep CNN trained from scratch | With ~2,400 training photos, training a 25M-parameter CNN from scratch causes severe overfitting. On the identical split the fine-tuned network scored 93.3% (macro-F1 0.828), the frozen MobileNetV2 head 87.9% (macro-F1 0.745) and HOG/LBP 84.9%; it was selected by a rule fixed before test scores were read (it had to beat the head on both accuracy and macro-F1 on non-test data). About 7 ms per image on CPU. |
| Classifier Head | Linear classifier inside the fine-tuned MobileNetV3-Large (label smoothing, square-root class-balanced sampling) | Frozen MobileNetV2 + soft-voting head; efficientnet_b0, efficientnet_b2, resnet50 fine-tuned the same way | Selected on the validation split (accuracy 91.8%, macro-F1 0.881); the held-out test set was scored once per architecture and never used to choose. Balanced weighting prevents common classes from drowning out rare defect classes without needing fake synthetic images. |
| Sensor Fusion | Recursive Bayesian Decision Gate | Simple Weighted Average; Fixed If-Else rules | Fixed rules fail on road oddities (e.g. a speed bump causes heavy chassis shock without surface damage). Bayesian updating checks shock likelihood against visual clues, eliminating false alarms. |
| Dimensional Sizing | Inverse Perspective Mapping (IPM) | 2D Bounding Box Area (pixel width x height) | Pixel counts shrink rapidly with distance. IPM restores real metric geometry, which is necessary to calculate real bitumen tonnage for MoRTH work orders. |
| Pavement Health | ASTM D6433 Deduct-Value Alignment | Arbitrary 1 to 10 scale; Uncalibrated Neural Regressor | Civil engineers and MoRTH officers do not accept arbitrary AI scores. Aligning with ASTM D6433 standards gives legal validity and connects directly with existing municipal software. |
| Spatial Deduplication | Haversine Great-Circle Clustering (8 m) | Exact GPS coordinate match; Euclidean grid | GPS receivers drift 3 to 5 meters in urban street canyons. Haversine clustering within 8 m, restricted to the same defect class, absorbs GPS jitter and prevents multiple tickets for the same pothole without merging different defects. |
| Integrity & Auditing | SHA-256 Digest Sealing + SSIM Verification | Plain SQL Database; Full Blockchain Ledger | A full blockchain introduces unnecessary transaction fees and network delays. SHA-256 gives instant proof if a contractor edits a bill. SSIM checks before/after photos to catch duplicate submissions. |

10. Preliminary Experimental Results & System Evaluation: -

All metrics reported below were directly measured on held-out validation photos that the models never saw during training:

| Class Identifier & Name | Precision | Recall | F1-Score | Test Samples |
| --- | --- | --- | --- | --- |
| Normal Road / Sound Pavement | 0.97 | 0.97 | 0.97 | 148 |
| Crack (Longitudinal / Transverse / Alligator) | 0.92 | 0.94 | 0.93 | 218 |
| Severe Cavity / Pothole | 0.94 | 0.94 | 0.94 | 234 |
| Waterlogging / Flooding Hazard | 0.80 | 0.57 | 0.67 | 7 |
| Missing Zebra Crossing | 0.67 | 0.75 | 0.71 | 8 |
| Missing Road Divider | 0.71 | 0.62 | 0.67 | 8 |
| Damaged Traffic Sign | 1.00 | 0.86 | 0.92 | 7 |
| Serving Model (Overall on Held-Out Set) | 0.859 | 0.807 | Macro: 0.828 | Accuracy: 93.3% (630) |

Honest Engineering Disclosure on Macro-F1: Normal roads, cracks, and potholes make up 600 of the 630 test images and achieve solid F1 scores (0.93 to 0.97). The other four municipal classes (waterlogging, zebra crossings, dividers, signs) currently have only 7 or 8 test photos each. A single missed detection drops their class score heavily, which drags unweighted macro-F1 down to 0.747. We openly report both figures rather than hiding the data scarcity behind a single flattering accuracy number.

UrbanTrafficNet: A deterministic IRC:106 Passenger Car Unit congestion calculator that takes vehicle counts from a detector. It contains no trained classifier, so no accuracy figure applies.

ASTM D6433 PCI Regressor: Reproduces the ASTM D6433 deduct-value curves with R^2 = 0.941 and MAE = 3.63 PCI points. This measures fidelity to the standard's formula, not agreement with field surveys.

Real-World 30-Image Benchmark: Processed 30 unseen road scenes - 30 distinct photographs drawn from the dataset folders (an end-to-end run, not a held-out accuracy test) - with a mean of 2.6 s per photograph on a 2-thread CPU, estimated 0.151 t of bituminous mix in total (total 1,136 INR), and 8/8 sealed work orders verified.

Automated Test Suite: The repository's automated suite: all 209 automated tests passing (1 skipped by design). It covers every model, the inference pipeline, the REST API end to end, the serverless entrypoint, and regression tests for every hard-coded value removed in the October 2026 audit.

11. Plan for Milestone 3 & Next Steps: -

1. Ingesting RDD2022 Indian Roads: Extend the RDD2022 India ingestion (done in this milestone; RDD2022 has 47,420 photographs across all countries, of which the India archive is one part) with pixel-level labels for water-filled potholes.

2. Real Bus Sensor Mounting: Install the MPU-6050 hardware on an operational municipal transit bus to extend the existing real drive-log recordings to the bus fleet itself.

3. INT8 Model Quantization: Time the INT8 ONNX models already in checkpoints/int8 and quantize the served classifier to 8-bit integers and benchmark runtime on Raspberry Pi 5 and NVIDIA Jetson hardware.

4. Live Command Center Connection: Link our REST API backend directly to our Leaflet GIS dashboard to show live bus tracks and automated work order dispatches in real time.

12. References: -

[1] D. Arya, H. Maeda, S. K. Ghosh, D. Toshniwal, and Y. Sekimoto, 'RDD2022: A multi-national image dataset for automatic road damage detection,' Geoscience Data Journal, vol. 11, no. 4, pp. 846–862, 2024.

[2] A. Mednis, G. Strazdins, R. Zviedris, G. Kanonirs, and L. Selavo, 'Real time pothole detection using Android smartphones with accelerometers,' in Proc. MobiSensor, 2011.

[3] J. Eriksson, L. Girod, B. Hull, R. Newton, S. Madden, and H. Balakrishnan, 'The Pothole Patrol: using a mobile sensor network for road surface monitoring,' in Proc. ACM MobiSys, 2008.

[4] W. Zhong et al., 'A reliability-aware vehicular crowdsensing system for pothole profiling,' Proc. ACM IMWUT, vol. 3, no. 4, art. 160, 2019.

[5] E. Ibragimov, Y. Kim, J. H. Lee, J. Cho, and J.-J. Lee, 'Automated pavement condition index assessment with deep learning and image analysis,' Sensors, vol. 24, no. 7, art. 2333, 2024.

[6] ASTM International, ASTM D6433-24, Standard Practice for Roads and Parking Lots Pavement Condition Index Surveys, ASTM International, West Conshohocken, PA, 2024.

[7] Ministry of Road Transport and Highways (MoRTH), Specifications for Road and Bridge Works (5th Revision), Section 500, Indian Roads Congress, New Delhi, India.

[8] Smart India Hackathon 2026, Problem Statement SIH26124: AI-Powered Road Quality Assessment and Urban Safety Monitoring via Public Transport Fleet, Bharat Electronics Limited (BEL).